[image: image1.jpg]






Condition monitoring for industrial gear units 
DriveRadar® IoT Suite instead of a crystal ball 
Bruchsal, Hannover Messe 2021: Under its DriveRadar® umbrella brand, SEW‑EURODRIVE is introducing the DriveRadar® IoT Suite condition monitoring system, which records and evaluates operating data from industrial gear units. This makes it possible to reliably determine the behavior of drives and system components in advance and preemptively plan maintenance and repair measures. 
With the launch of DriveRadar® IoT Suite for industrial gear units, SEW‑EURODRIVE is offering a condition monitoring service that digitally records and evaluates operating data, creating forecasts of status changes. A coordinated package of sensor technology continuously records variables such as ambient temperature, gear unit oil temperature, input speed, oil levels, and the vibration behavior of rolling bearings and gearing. 
An edge processing unit (EPU) records, stores and consolidates this data. It then uses a mobile network to send the measured values to the SEW‑EURODRIVE data center in encrypted form, where they are evaluated and interpreted by the system. The DriveRadar® IoT Suite web application gives customers the option of viewing the status of all monitored gear units as well as all processed data. They can also be notified directly of any status changes via a smartphone app designed for DriveRadar® IoT Suite services. 
This means customers can determine the status of individual rolling bearings and their components (outer ring, inner ring, rolling elements) or predict when the next oil change will be required, for example. As a result, it is possible to plan maintenance and repairs for industrial gear units in advance and prevent failures.
DriveRadar® IoT Suite for industrial gear units is available pre-installed on new Generation X.e helical and bevel-helical gear units. However, retrofitting is also possible. Industrial gear units that are already installed can be equipped for monitoring in Drive Radar® IoT Suite during a service call either at the customer's premises or at SEW. 
Key word for reader inquiries: "DriveRadar® IoT Suite for industrial gear units" 
The "DriveRadar for industrial gear units" image is part of this press release. 
For more information, go to: www.sew-eurodrive.de/driveradar-ig 
Valid until: April 1, 2022 
Please contact us prior to publication after this date. 

Contact for editors: 





SEW�EURODRIVE GmbH & Co KG 


Corporate Communication


Ernst-Blickle-Strasse 42 


76646 Bruchsal�Germany 


� HYPERLINK "http://www.sew-eurodrive.de" ��http://www.sew-eurodrive.de�





Mr. Stefan Brill


Press Spokesperson 


Tel.: +49 7251 75-2525 


Fax: +49 7251 75-502525 


�HYPERLINK "mailto:stefan.brill@sew-eurodrive.de"��stefan.brill@sew-eurodrive.de� 


�
Submit reader questions to: 





SEW�EURODRIVE GmbH & Co KG 


Media and Public Relations 


Ernst-Blickle-Strasse 42 


76646 Bruchsal�Germany 


� HYPERLINK "http://www.sew-eurodrive.de" �http://www.sew-eurodrive.de�  





Tel.: +49 7251 75-0 


� HYPERLINK "mailto:sew@sew-eurodrive.de" ��sew@sew-eurodrive.de� �
�
Reprint requested  –  Use free of charge  –  Submit free copy �
�










